Ziya Gökalp Mah. İkitelli Organize Sanayi Bölgesi Atatürk Bulvarı No:50 34490 Başakşehir / İstanbul / TÜRKİYE

Telefon : +90 212 549 22 52

Faks      : +90 212 549 25 80

Web      : www.emasas.com.tr

EMAS ELEKTROTEKNİK MAKİNA    SANAYİ VE TİCARET A.Ş

Technical Specifications : P2AB

Technical Information :

Body:. Plastic Body.. Opportunity of positioning with plates and Badge Bases.. General Specifications. Operating Temparature; -5/+40°C. Mechanical life; 0,5 Million (3000 open-close/hour. Electrical Life; 0,1 Million (1800 open-close/hour). C.Block Tightening Torque; 0,6-0,8 Nm. C.Block Connection Wire Profile; Min: 1x0,5 mm'2. Wire Profile; Max: 2x2,5 mm'2. Protection Degree; IP50. Notes:Technical specifications of the contact block mounted in the control boxes

Standards: - TS EN 60947 -5-1 In contact Blocks; - VDE 0660 - UL 508

| General Specifications |  |  |
| --- | --- | --- |
| Range Of Use |  | AC15 |
| Operating Voltage | Ue | 250V AC |
| Fregquency | f | 50Hz |
| Operating Current | Ie | 4A |
| Insulation Resistance | Ui | 300V |
| Stroke Resistance Voltage | Uimp | 2.5kV |
| Dielectrical Strength |  | 1500V AC |

Electrical Informations :